1995 Jan. 19

HOW TO USE ART FUNCTION

H. Hara

ARS and ART functions independently work. One does not interfere another.

ART function is independent of the morning patrol.

<< procedure >>

1. Change the ART ENA/DIS in the COMMON table into ENA. ART TFS/IRU in

the COMMON table should be IRU. TFS must not be used because of a

part of failure of TFSS.

2. Set the observing coordinates in the ART slots of the OBSERVING table,

and the select the PFI region in PFI SEQ tables.

3. Send the SXT table to the satellite.

4. Send `SXT CTL MANU' to the satellite.

5. Send the SXT table to the satellite by the command of `SXT TABLE WR'.

6. After sending `SXT TABLE RE', KSC people should make sure of the new

coordinates in the ART slots looking at the QL (third page [PF15] of

SXTQLCC) display, NOT using SXTSPT. The third page of the QL is almost

the same as the OBSERVING table of SXTSPT.

7. Send `ART INIT' command. The new coordinates are copied to the reference

coordinates (right portion in the ART slots). This should be also made

sure by KSC people looking at the QL.

8. Send SXT CTL AT.